GUIDELINES — FOR PROJECTS WHERE THE EXISTING SHOULDER IS OUT OF COMPLIANCE WITH THE ROAD AND BRIDGE STANDARDS EITHER DUE TO PLANT MIX OVERLAY WORK OR BECAUSE THE EXISTING SHOULDER WAS ALREADY OUT OF COMPLIANCE.  THIS INCLUDES SHOULDER WORK IN AREAS SCHEDULED FOR NEW GUARDRAIL, GUARDRAIL IMPROVEMENTS OR GUARDRAIL REPLACEMENTS.

S305EM1-1215

VIRGINIA DEPARTMENT OF TRANSPORTATION

SPECIAL PROVISION FOR

SHOULDER RENOVATION

December 9, 2015

I.	DESCRIPTION

This work shall consist of renovating existing low (erosion or overlay) and high shoulders (debris buildup) and shoulders disturbed due to plant mix overlay and/or guardrail work as specified in the Contract to provide finished shoulder designs and guardrail heights that conform to the applicable Road and Bridge Specifications, Road and Bridge Standards, and these specifications.  For the purposes of this provision, machining shoulders and manual shoulder restoration shall be viewed as placing, grading, and compacting operations of approved shoulder materials performed by mechanized equipment or manually.  Materials allowed for renovating shoulders shall include furnishing and delivery of these materials to the jobsite or to the location(s) designated in the contract documents.

II.	MATERIALS

Allowable shoulder materials shall conform to the following:

Aggregate base material (type and size as specified) shall be virgin material conforming to Section 208 of the Specifications.

Alternate Shoulder Material may include Recycled Asphalt Pavement Material (RAP), non-fractionated Crushed Hydraulic Cement Concrete (CHCC), or a blend of virgin aggregate base material and RAP or CHCC as requested for use.  The use of Alternate Shoulder Material is permissive and subject to the limitations as described herein and elsewhere in the Special Provision Copied Note for Alternate Category Bid Items and Award of Contract included in the Contract.

Alternate Shoulder Material, if used, shall be 1-inch maximum size as determined visually or by field measurement.  Alternate Shoulder Material shall have a loose, unconsolidated consistency and shall not contain any clusters of materials that exceed the 1-inch grading requirement.  Material out of conformance with the maximum size limitation will be rejected.

Blended material, if used, shall be thoroughly mixed (manipulated) and shall have a dappled appearance when placed, graded, and compacted.

III.	PROCEDURES

The following general procedures shall apply to shoulder renovation work:

The use of more than one type of approved material on uninterrupted runs of shoulder work will not be permitted.

The use of CHCC or any CHCC blended material as Alternate Shoulder Material will only be permitted in areas meeting the following conditions:  No more than 3 feet in width and no more than 3 inches in compacted depth once placed.

Shoulder material shall be spread, graded, and compacted in accordance with the requirements of Section 305.03(e) of the Specifications, except as noted herein.  Subgrade shaping will generally not be required unless directed by the Engineer.  However, when shaping of the subgrade is required, the cost of such work shall be included in the cost of machining shoulders or manual shoulder renovation work.

The maximum compacted lift thickness of Aggregate Base Material or Alternate Shoulder Material (except CHCC or CHCC blends) shall be 6 inches.  The acceptability of furnished and finished (compacted) shoulder material will be determined by visual inspection, field measurement, or a combination thereof, at the discretion of the Engineer.

Final pavement surface edge or final paved or stabilized shoulder surface edge shall include existing pavement not designated for overlay and completely compacted pavement overlays, and their corresponding shoulders.

The Contractor shall promptly remove and dispose of surplus shoulder material encountered as a result of shoulder renovation work as well as any shoulder material spilled, left or tracked on the pavement.

Grading for shoulder renovation shall be performed by the following methods:

- Machining shoulders shall be performed in areas where there is no existing guardrail and none is scheduled to be placed or updated under this contract, or in areas with existing guardrail where that guardrail will not be disturbed.  Machining shoulders shall also be performed in areas scheduled for new guardrail installation before new guardrail is installed, or in areas where existing guardrail will be removed in preparation for guardrail improvement or guardrail replacement.  In each of these grading situations it is to be assumed grading can be performed by mechanized equipment unencumbered by existing or newly installed guardrail.

Machining shoulders shall include grading shoulders to appropriate slope and grade where sufficient material is present to renovate the existing shoulders, grading existing shoulders to fill in low areas after allowable shoulder material has been placed, or grading down areas where high shoulders exist due to debris buildup.

Machining shoulders shall result in a uniformly finished slope to the shoulder break that conforms to the applicable Road and Bridge Standards (See included sketch) after compaction.  Renovated shoulders shall smoothly tie the graded shoulder edge elevation to the adjoining elevation of the final pavement surface edge and final paved or stabilized shoulder surface edge.

- Manual shoulder restoration shall be used to renovate shoulders in areas where existing guardrail will be undisturbed by adjacent plant mix or other operations specified in the contract.

Manual shoulder restoration shall include grading shoulders around existing guardrail by hand or other intensive production methods to appropriate slope and grade where sufficient material is present to restore the existing shoulders, grading existing shoulders to fill in low areas after allowable shoulder material has been placed, or grading down areas where high shoulders exists due to debris buildup.

Manual shoulder restoration shall result in a uniformly finished slope to the shoulder break that conforms to the applicable Road and Bridge Standards (See included sketch) after compaction.  Restored shoulder work shall smoothly tie the graded shoulder edge elevation to the adjoining elevation of the final pavement surface edge and final paved or stabilized shoulder surface edge.

Note: Aggregate base material is the only allowable material for manual shoulder restoration work unless otherwise approved by the Engineer.

Allowable shoulder materials (depending on the type of shoulder renovation operation specified in the contract or directed by the Engineer) shall be furnished and placed by the Contractor in low shoulder areas, then machined or manually graded off as necessary, and then compacted to provide a finished cross slope that conforms to the applicable Road and Bridge Standards as well as the existing road profile grade.

Where guardrail is to be installed or reinstalled, the placement widths and limits of allowable shoulder materials shall be in accordance with the detail requirements for the specific type of guardrail as designated in the contract documents and as shown in the Road and Bridge Standards, the attached sketch, or as otherwise indicated in the contract documents.  Guardrail height shall conform to the Road and Bridge Standards for the applicable guardrail type once work is completed.  This work shall proceed as directed by the Engineer.

IV.	MEASUREMENT AND PAYMENT

Machining shoulders will be measured in linear feet along the adjacent edge of pavement and will be paid for at the contract unit price per linear foot.  The price shall include placing, grading, and compaction.  This price shall also include removing and disposing of surplus, spilled, and tracked material resulting from the Contractor’s operations.

Manual shoulder restoration will be measured in linear feet along the adjacent edge of pavement specified in the contact documents or directed by the Engineer, and will be paid for at the contract unit price per linear foot.  The price shall include placing, grading, and compaction.  This price shall also include removing and disposing of surplus, spilled, and tracked material resulting from the Contractor’s operations.

Aggregate base material will be measured in tons and will be paid for at the contract unit price per ton.  The price bid shall include furnishing and delivery.

Alternate Shoulder Material, if requested and authorized for use, will be measured in tons and will be paid for at the contract unit price per ton as bid in Section 0002 of the Special Provision Copied Note for Alternate Category Bid Items and Award of Contract.  The price bid shall include furnishing and delivery.

Tonnage for Alternate Shoulder Material will be based on certified weigh tickets from the source of supply, or when supplied directly from the field, will be computed on the basis of 110 pounds per inch of depth per square yard, converted to tons.

Payment will be made under:

| Pay Item | Pay Unit |
| --- | --- |
| Aggregate Base Material, Type ( ), No. ( ) | Ton 16242 |
| Machining Shoulders | Linear Foot 16387 |
| Manual Shoulder Restoration | Linear Foot 16398 |
| Alternate Shoulder Material | Ton |