GUIDELINES — USE ON PROJECTS WHERE DEMOLITION OF STRUCTURES CONTAINING NON-FRIABLE ASBESTOS-CONTAINING MATERIALS (ACM) IS REQUIRED; ALSO, USE WITH THE SPECIAL PROVISION FOR ASBESTOS REMOVAL FOR ROAD CONSTRUCTION PROJECTS.  IF AN ASBESTOS REPORT IS AVAILABLE, A COPIED NOTE SHOULD BE CREATED DETAILING THE LOCATIONS, NESHAP CATEGORY, CONDITION, AND APPROXIMATE QAUNTITIES OF ASBESTOS.

S516C00-0416

VIRGINIA DEPARTMENT OF TRANSPORTATION

SPECIAL PROVISION FOR

DEMOLITION OF STRUCTURES CONTAINING

NON-FRIABLE ASBESTOS CONTAINING MATERIALS

November 19, 2015

I.	DESCRIPTION

This work shall include the demolition of structures containing non-friable asbestos-containing materials (ACM) and in which all Regulated Asbestos-Containing Materials (RACM) have been removed. This special provision complements the Special Provision for Asbestos Removal for Road Construction Projects and Section 516 of VDOT Road and Bridge Specifications.

ACM that may be present in the structures to be demolished may include Category I Non-friable ACM in “good condition” (e.g. resilient flooring, asphalt roofing products, mastics), and certain Category II Non-friable materials (e.g. floor tile mastics) that do not have a high probability of becoming or will not become crumbled, pulverized or reduced to powder by the forces expected to act on the material in the course of demolition.

II.	LIMITATIONS

1.	The Demolition Contractor shall not be financially affiliated with the Project Monitor on the project. The project monitoring services shall be directly contracted for by VDOT and the monitoring personnel shall be accountable only to the Department.

2.	The Contractor’s liability insurance or bonding shall not contain any pollution exclusion type of provisions.

3.	No demolition of structures shall be performed by intentional burning when any amount of ACM is present in the structure.

III.	DEFINITIONS

1.	ADEQUATELY WET: means to sufficiently mix or penetrate with liquid to prevent the release of particulates. If visible emissions are observed coming from any asbestos containing material, then the material has not been adequately wet.

2.	AIR MONITORING: The process of measuring the fiber content of a specific volume of air in a stated period of time.

3.	ASBESTOS: The term asbestos includes Chrysotile, Amosite, Crocidolite, Tremolite, Anthrophyllite and Actinolite, and any of these minerals that have been chemically treated and/or altered.

4.	ASBESTOS-CONTAINING MATERIAL (ACM): Material or product containing more than 1% asbestos.

5.	CATEGORY I NON-FRIABLE ACM: Asbestos-containing packings, gaskets, resilient floor covering and asphalt roofing products containing more than one percent asbestos as determined using the method specified in Appendix A, subpart F, 40 CFR Part 763, Section 1, Polarized Light Microscopy.

6.	CATEGORY II NON-FRIABLE ACM: Any material, excluding Category I non-friable ACM, containing more than one per cent asbestos as determined using the specified in Appendix A, subpart F, 40 CFR Part 763, Section 1, Polarized Light Microscopy -containing material, that, when dry, cannot be crumbled, pulverized, or reduced to powder by hand pressure.

7.	CUTTING: To penetrate with a sharp-edged instrument and includes sawing, but does not include shearing, slicing, or punching.

8.	FRIABLE ACM: Any material containing more than one percent asbestos as determined by polarized light microscopy that when dry, can be crumbled, pulverized, or reduced to powder by hand pressure.

9.	DEMOLITION: The wrecking or taking out of any load-supporting structural member of a facility together with any related handling operations or the intentional controlled burning of a facility.

10.	GRINDING: To reduce to powder or small fragments and includes mechanical chipping or drilling.

11.	IN POOR CONDITION: The binding of the material is losing its integrity as demonstrated by peeling, cracking or crumbling of the material.

12.	PROJECT MONITOR: Individual licensed by the Virginia Department of Professional and Occupational Regulation to observe and monitor the activities of the demolition contractor to determine that proper work practices are used and compliance with all federal, state and local laws and regulations are maintained.

13.	REGULATED ASBESTOS-CONTAINING MATERIAL (RACM): Includes (1) friable asbestos material; (2) Category I nonfriable ACM that has become friable; (3) Category I nonfriable ACM that will be subjected to sanding, grinding, cutting or abrading, or (4) Category II nonfriable ACM that has a high probability of becoming or has become crumbled, pulverized or reduced to powder by forces expected to act on the material in the course of demolition or renovation operations.

14.	VISIBLE EMISSIONS: Any emissions coming from asbestos-containing waste material that are visually detectable without the aid of instruments.

15.	WASTE SHIPMENT RECORD: A shipping document required to be originated and signed by the waste generator and used to track and substantiate the disposition of asbestos containing waste material.

IV.	NOTIFICATIONS

1.	Contractors also performing any demolition activities shall provide the requisite 10-day demolition notifications pursuant to § 61.145(b), irrespective of any minimum quantity or other exclusions. Notifications should be addressed to:

Virginia Department of Labor and Industry

Asbestos Program

Powers-Taylor Building

13 South Thirteenth Street

Richmond, VA 23219

Land and Chemical Division

EPA Region III

LC62

1650 Arch Street

Philadelphia, PA 19103

2.	The Contractor shall give a three (3) full business day notification to the Asbestos Project Monitor, the VDOT Area Construction Engineer, and the VDOT Project Manager prior to beginning work.

V.	WORK PRACTICES

1.	The Contractor shall keep all demolition materials and debris adequately wet so as to prevent the release of air borne particulates throughout the demolition and loading operations.

2.	The Contractor shall perform demolition of structures in accordance with the requirements of Section 516 of the VDOT Road and Bridge Specifications and this special provision.

3.	The Contractor shall perform demolition of structures containing non-friable asbestos-containing materials in accordance with all applicable EPA, OSHA and VOSHA regulations, and shall follow EPA and OSHA workplace guidelines unless they are shown to not be applicable. EPA workplace guidelines include (1) EPA regulations 40 CFR Part 61 Subparts A and M and (2) “Demolition Practices Under the Asbestos NESHAP” (TRC Environmental Corporation Work Assignment No. IA2-19). OSHA workplace guidelines include any currently applicable OSHA compliance directives or instructions. In any instance of conflict between the VOSHA and OSHA requirements, the VOSHA requirements shall take precedence. Any “de minimus” exemptions or reduced requirements for demolition activities involving certain minimum quantities of non-friable ACM that may be provided in the aforementioned references shall not apply to VDOT demolition activities.

4.	No sanding, grinding, cutting or abrading of any non-friable ACM shall be allowed prior to or during demolition.

5.	Where possible, the Contractor shall separate concrete and other masonry materials from other demolition materials and legally dispose of the debris separately.

6.	Segregating materials by intentionally running over debris with tractor treads will not be permitted (e.g. separation of flooring from concrete).

7.	Waste consolidation efforts involving the use of pneumatic-hammers or other mechanical devices such as grinders to break up materials covered or coated with Category I non-friable ACM is prohibited.

8.	Consolidating debris containing Category I non-friable ACM in the basement of a building and subsequently grinding or crushing is prohibited.

9.	The use of cranes with clamshells or other heavy machinery with rakes or buckets to partially reduce debris-containing Category I non-friable ACM is permissible provided the material remains recognizable in its original form.

10.	Concrete to which asbestos-containing resilient flooring is attached shall not be broken down using concrete-pulverizing machinery.

11.	VDOT shall provide, at its discretion and cost, project monitoring that may include collection of air samples to ensure that demolition practices are performed in accordance with this special provision. The Contractor shall cooperate with the Project Monitor in all testing and sampling activities. The Project Monitor shall inform the Contractor of (1) any area samples outside of the containment with results in excess of 0.01 fiber/cubic centimeter and (2) any instance where the Contractor is using demolition methods prohibited by this section (the project Monitor shall also notify the VDOT Engineer). Where fiber counts are determined to exceed this standard, the Contractor shall immediately discontinue operations until the cause of the excursion can be identified and corrected. Where prohibited demolition practices are observed, the Contractor shall immediately discontinue such operations and employ proper work practices. The Contractor shall, at no additional expense to the Department, be responsible for the costs of any standby time and/or corrective measures if fiber counts exceed the standard or improper demolition practices are employed.

VI.	DISPOSAL

1.	The Contractor shall provide notification to the landfill that the debris contains non-friable/non-regulated ACM and shall only dispose of debris containing non-friable ACM in a permitted landfill that provides daily soil cover.

2.	The Contractor shall remove, transport, and dispose of ACM from the job site in accordance with Virginia Department of Environmental Quality (VDEQ) regulations and other applicable federal, state, and local regulations.

3.	The debris shall be transported in covered DOT-approved containers.

4.	The Contractor shall be responsible for generating and maintaining waste shipment records, irrespective of whether the ACM-containing debris is, or is not, RACM.

5.	Within thirty-five (35) days of the deposit of a load of waste at the designated landfill, the Contractor shall submit a copy of the certificate of disposal from the landfill to the Engineer. VDOT must receive all acceptable waste manifests and certificates of disposal prior to making any payments to the Contractor.

VII.	MEASUREMENT AND PAYMENT

Demolition with ACM will be paid for at the contract lump sum price per structure. This price shall include coordinating and performing all work associated with disconnecting utilities, employing appropriate demolition and loading practices, disposal of materials and cleaning up. The Contractor shall take into consideration any salvage value of any material removed and shall include the same in the lump sum bid price.

Payment will be made under:

| Pay Item | Pay Unit |
| --- | --- |
| Demolition with ACM (Structure) | Lump Sum |