GUIDELINES — PROJECTS REQUIRING TRAFFIC SIGNS,

SS70103-0815                                                                                                                   April 15, 2015

VIRGINIA DEPARTMENT OF TRANSPORTATION

2007 ROAD AND BRIDGE SUPPLEMENTAL SPECIFICATIONS

SUPPLEMENTAL SECTION 701—TRAFFIC SIGNS

SECTION 701—TRAFFIC SIGNS of the Specifications is amended as follows:

Section 701.03—Procedures is amended as follows:

Section 701.03(a)2. Sign panels is replaced with the following:

Panels for permanent signs shall be fabricated of aluminum 0.100 inch in thickness and shall be smooth, flat, and free of metal burrs and splinters.  Sign panels for overlays shall be aluminum alloy conforming to the Section 229.02(a), between 0.080 and 0.100 gage in thickness.

Extruded sign panels shall conform to the VDOT Road and Bridge Standards and Section 229.02(c).

Section 701.03(a)5. Joining sign base panels is amended to replace the first sentence with the following:

Horizontal joints shall be constructed according to VDOT Standard Drawing SPD-1.

Section 701.03(d) Erection is amended to replace the first sentence of the first paragraph with the following:

The Contractor shall install sign panels on overhead sign structures so that the vertical clearance is not less than 19 feet and not more than 21 feet from the bottom of the lowest mounted sign panel to the crown of the roadway, unless otherwise specified on the plans.

Section 701.03(d). Erection is amended to delete the last sentence of the first paragraph.

Section 701.03(d). Erection is amended to delete the last paragraph.

Section 701.03(d). Erection is amended to include the following:

Overlay panels shall be installed on a flat portion of the existing sign panel with no protruding bolts or bolt heads.  Overlay of overhead sign panels shall be according to details shown on the plans.

Section 701.04 Erection is amended to replace the first paragraph with the following:

Sign panels will be measured in square feet and will be paid for at the contract unit price per square foot.  This price shall include background sheeting, sign messages, finishing, framing units, hanger assemblies, bracing, stiffeners, splicing, backing strips, post clips/post clamps, warranty and labeling.