SU512002A 512 Maintaining Traffic

GUIDELINES — ALL PROJECTS WITH GUARDRAIL AND IMPACT ATTENUATORS.

VIRGINIA DEPARTMENT OF TRANSPORTATION

SPECIAL PROVISION FOR

SECTION 512—MAINTAINING TRAFFIC

March 18, 2016a

SECTION 512—MAINTAINING TRAFFIC of the Specifications is amended as follows:

Section 512.03(a) Signs is amended to replace the last paragraph with the following:

When construction signs are covered to prevent the display of the message, the entire sign shall be covered with silt fence or other materials approved by the Engineer such that no portion of the message side of the sign shall be visible.  Plywood shall be used on ground-mounted construction signs only.  Attachment methods used to attach the covering material to the signs shall be of a durable construction that will prevent the unintentional detachment of the material from the sign.  At no times shall a construction sign and/or post be rotated to prevent the display of the message.  In addition, the posts where the signs are being covered shall have two ED-3 Type II delineators mounting vertically on the post below the signs at a height of 4 feet to the top of the topmost delineator.  The bottom delineator shall be mounted 6 inches below the top delineator.

Section 512.03(b) Flagger Service and Pilot Vehicles is amended to replace the last paragraph with the following:

Portable traffic control signals conforming to the requirements of Section 512.03(h)2 of the Specifications may be used in lieu of flagger service when specified or approved by the Regional Traffic Engineer.  When portable traffic control signals are used in lieu of flagger service, the portable traffic control signals will be measured and paid for separately.

Section 512.03(e)b. Group 2 devices is amended to replace the first paragraph with the following:

b.	Group 2 devices shall be drums or vertical panels.  Drums shall be round, or partially round with no more than one flat side; made from plastic; have a minimum height of 36 inches, have a cross-sectional width no less than 18 inches in any direction; and conform to the requirements of the Virginia Work Area Protection Manual.  Drums shall be designed to allow for separation of ballast and drum upon vehicular impact but not from wind and vacuum created by passing vehicles.  Drums of two-piece design, i.e., drum and associated base, shall utilize sufficient amounts of enclosed sand at the base in accordance with the manufacturer’s recommendations to provide stable drum support.  The base shall be not greater than 5 inches in height.  Two-piece drums may also utilize a flared drum foundation and collar of not more than 5 inches in height and of suitable shape and weight to provide stable support.  One-piece drums may be used provided they comply with these above requirements.

Section 512.03(f)1 Guardrail service and terminal treatments is amended to include the following:

All end terminals used in conjunction with guardrail barrier service shall be from manufacturers on the Materials Division’s Approved Products List 12 and the FHWA NCHRP 350 or MASH approved list linked in List 12.  New Guardrail Terminals (GR-7 & GR-9) shall be permanently identified in a location readily visible for inspection that is not susceptible to damage by die stamping or engraving.  The identification shall include Manufacturer, Date and Site of Manufacture, and Model Number.

Section 512.03 (g) Impact Attenuator Service is amended to include the following:

New impact attenuators shall be permanently identified in a location readily visible for inspection that is not susceptible to damage by die stamping or engraving.  The identification shall include Manufacturer, Date and Site of Manufacture, and Model Number.

Section 512.03 (p) Truck-mounted Attenuators is amended to include the following:

New truck-mounted and trailer-mounted attenuators shall be permanently identified in a location readily visible for inspection that is not susceptible to damage by die stamping or engraving.  The identification shall include Manufacturer, Date and Site of Manufacture, and Model Number.

Section 512.03 Procedures is amended to add (r) Work Zone Traffic Control as the following:

(r)	Work Zone Traffic Control: The Contractor shall provide individuals trained in Work Zone Traffic Control in accordance with the requirements of Section 105.14 of the Specifications.

Section 512.04 Measurement and Payment is amended to add the following:

Basic Work Zone Traffic Control – Separate payment will not be made for providing a person to meet the requirements of Section 105.14 of the Specifications.  The cost thereof shall be included in the price of other appropriate pay items.

Intermediate Work Zone Traffic Control - Separate payment will not be made for providing a person to meet the requirements of Section 105.14 of the Specifications.  The cost thereof shall be included in the price of other appropriate pay items.

Section 512.04 Measurement and Payment is amended to replace the pay item and corresponding pay unit for “Eradication of existing pavement markings” with the following:

Eradication of existing pavement markings will be measured in linear feet of a 6-inch width or portion thereof as specified herein.  Widths that exceed a 6-inch increment by more than 1/2 inch will be measured as the next 6-inch increment.  Measurement and payment for eradication of existing pavement markings specified herein shall be limited to linear pavement line markings.  Eradication of existing pavement markings will be paid for at the contract unit price per linear foot.  This price shall include removing linear pavement line markings and disposing of residue.

Eradication of existing nonlinear pavement markings will be measured in square feet based on a theoretical box defined by the outermost limits of the nonlinear pavement marking.  Nonlinear pavement markings shall include but not be limited to stop bars, arrows, images and messages.  Eradication of existing nonlinear pavement markings will be paid for at the contract unit price per square foot.  This price shall include removing nonlinear pavement markings and disposing of residue.

Payment will be made under:

| Pay Item | Pay Unit |
| --- | --- |
| Eradication of existing pavement marking | Linear foot 54105 |
| Eradication of existing nonlinear pavement marking | Square foot 54106 |