SS235-002020-01	May 6, 2022

VIRGINIA DEPARTMENT OF TRANSPORTATION

2020 ROAD AND BRIDGE SUPPLEMENTAL SPECIFICATIONS

SECTION 235 – RETROREFLECTORS

SECTION 235 – RETROREFLECTORS of the Specifications is deleted and replaced as follows:

## 235.01 – Description

Retroreflectors are retroreflective surfaces that redirect the vehicle headlights back to the driver to delineate the road. The retroreflective surface may consist of a plastic prismatic reflector or retroreflective sheeting.  Retroreflectors are used with:

- Pavement Markers (Permanent and Temporary)

- Delineators (Guardrail, Barrier, Flexible Post, Road Edge)

Pavement markers and Delineators shall be approved by reviewing performance data from one or both of the following test programs:

- AASHTO’s National Transportation Product Evaluation Program (AASHTO/NTPEP). Test data values used for approval may be based upon the data generated per the applicable NTPEP Work Plan.

- VDOT Test Facility – VDOT may elect to evaluate performance from their own test facility.

## 235.02 – Detail Requirements

- Inlaid Pavement Markers – Holders for inlaid pavement markers shall be made of polycarbonate plastic nominally 4.75 inches wide excluding breakaway tabs,  and shall be able to hold retroreflectors from the Department’s Approved List 22 under Inlaid Pavement Markers.The top of the the retroreflector shall be 1/8 inch below the pavement surface when installed with the breakaway positioning tabs resting on the pavement surface.

Retroreflectors for inlaid pavement markers shall have a nominal width of 4 inches excluding the holders.

- Pavement Markers (Temporary) – Refer to VTM-70 for testing and approval

- Pavement Markers (Permanent) – Refer to VTM-70 for testing and approval

- Delineators – Refer to VTM-70 for testing and approval

- Aluminum panels for delineators shall be at least 0.064 inch thick conforming to ASTM B-209, alloy 5052.